# Appendix D. Raw Experimental Data

This appendix records raw structural data for both designs. The Snake baseline counts below are transcribed from the supplied static analysis of snake_game_final.circ. The proposed CPU component and ROM tables below come from the uploaded 16bit_risc_cpu_final_3.circ. These sources use different overall counting scopes, so total inventories are reported separately; selected component classes are compared in Sections 14–16.

| Resource | Baseline count | Details / counting rule |
| --- | --- | --- |
| Registers | 15 blocks | 14 × 4-bit registers + 1 × 1-bit register = 57 stored state bits. |
| Standalone flip-flops | 0 blocks; 57 bits | No standalone flip-flop modules in the schematic; if register internals are counted, state storage totals 57 bits. |
| Adders | 3 | Three 4-bit adders. |
| Multipliers | 0 | No multiplier components. |
| Comparators | 27 | 23 four-bit coordinate/collision comparators + 4 seven-bit keyboard ASCII comparators. |
| Multiplexers | 27 | Standalone MUX count. If grouped with 6 demultiplexers and 6 decoders as selection-related devices, the group totals 39. |
| Dedicated counters | 0 | No dedicated Logisim Counter modules; 2 LFSR pseudo-random generators are counted separately. |
| RAM / ROM | 0 | No separate RAM/ROM chips; the listed game state is stored in registers. |
| Logic gates | 37 (38 incl. buffer) | 19 OR + 12 AND + 6 NOT = 37 primitive gates; one controlled buffer adds one if grouped. |
| Other functional units | 18 | 2 subtractors, 6 demultiplexers, 6 decoders, 2 LFSR generators, 1 keyboard module, and 1 16×16 dot-matrix display. |
| Clock | 1 | Included in the 128 functional-unit total. |
| Total functional units | 128 | 15 registers + 3 adders + 27 comparators + 27 MUX + 37 gates + 18 other units + 1 clock = 128. Multipliers, dedicated counters, and RAM/ROM count as zero. |
| Excluded support objects | 6 extenders; 7 splitters; 67 constants; 364 tunnels | Listed separately and excluded from the 128 functional-unit total. |
| Baseline timing / grid | 16×16; 8–16 updates/s noted | At 8 Hz, the clock-tick interval is 125 ms. The 10–15 gate-level logic depth is an estimate, not a measured physical delay. |

Source note: these baseline resource values were supplied from the analysis of snake_game_final.circ. I have preserved the source counting distinctions (standalone flip-flop blocks versus register state bits; MUX alone versus grouped selection devices; dedicated counters versus LFSRs) instead of combining alternate conventions into one count.

| Subcircuit | Components | Wires |
| --- | --- | --- |
| main | 121 | 665 |
| ALU | 25 | 124 |
| RegisterFile | 44 | 346 |
| ControlUnit | 33 | 159 |
| PC | 16 | 37 |
| Total functional subcircuits | 239 | 1331 |
| Empty placeholder | 0 | 0 |

Raw opcode counts from the embedded ROM in 16bit_risc_cpu_final_3.circ:

| Opcode | Mnemonic | Instruction type | Word count | Percent of 870 words |
| --- | --- | --- | --- | --- |
| 0x0 | NOP/HALT | Special | 76 | 8.74% |
| 0x1 | MOVI | I-type | 29 | 3.33% |
| 0x2 | MOVR | R-type | 14 | 1.61% |
| 0x3 | LW | I-type | 1 | 0.11% |
| 0x4 | SW | I-type | 11 | 1.26% |
| 0x5 | ADD | R-type | 40 | 4.60% |
| 0x6 | SUB | R-type | 16 | 1.84% |
| 0x7 | AND | R-type | 0 | 0.00% |
| 0x8 | ADDI | I-type | 334 | 38.39% |
| 0x9 | SUBI | I-type | 50 | 5.75% |
| 0xA | SWI | I-type | 85 | 9.77% |
| 0xB | CMP/BEQ | I-type | 67 | 7.70% |
| 0xC | SHL | R-type | 0 | 0.00% |
| 0xD | JMP/JAL | J-type | 147 | 16.90% |

Instruction type distribution calculated from the same 870 ROM words:

| Instruction type | Word count | Percentage |
| --- | --- | --- |
| I-type | 577 | 66.32% |
| J-type | 147 | 16.90% |
| R-type | 70 | 8.05% |
| Special | 76 | 8.74% |

Validation note: the uploaded circuit contains 870 ROM words; 12 of 14 defined opcode values are present, and opcode 0x0 appears 76 times. The separate testing_and_results_report.md frequency table sums to 866 entries while listing 72 NOP/HALT entries. Its static counts and estimated timing therefore remain separate from the 870-word parse above until both analyses are rerun against the same final circuit and ROM. A separate configuration note lists 1 kHz for Snake and maximum simulator frequency for Space Shooter, while the separate testing summary records 2.048 MHz; identify the actual settings used for each run before reporting runtime comparisons.

Supplementary values recorded in the separate testing summary (static counts or estimates, not direct runtime measurements):

| Raw metric | Recorded value | Basis / qualification |
| --- | --- | --- |
| General-purpose registers referenced | 16 of 16 | Static ROM analysis in the separate testing summary. |
| Most-written destination register | R3: 176 writes | Static decoded instruction count. |
| Most-read source register | R0: 221 reads | Static decoded instruction count. |
| JMP/JAL instructions | 147 | Static decoded instruction count. |
| CMP/BEQ branch instructions | 67 | Static decoded instruction count. |
| Unique jump targets | 110 | Distinct target addresses reported by the separate analysis. |
| MMIO load/store instructions | 97 | Count reported by the separate static analysis. |
| Simulation frequency in that report | 2,048,000 Hz (2.048 MHz) | Configuration recorded in the separate testing summary; verify against the actual run. |
| Estimated single-pass time | Approximately 423 µs | Estimate in the separate report, not a measured wall-clock runtime. |
| Estimated critical path | Approximately 11–13 gate-level delays | Logic-depth estimate; not a measured physical delay. |
| Reported worst-case instruction path | LW | The report identifies load-word as traversing the longest path. |